| Nominal Voltage | 51.2V |
| --- | --- |
| Rated Capacity | 200AH @0.2C to 40V |
| Stored energy | 10240Wh |
| Cycle life@100%DOD | >3000 Cycles@100%DOD >4000 Cycles@80%DOD >7000 Cycles@50%DOD |
| Approx. Weight | 78 kg |
| Internal Resistance | ≤60.0 mΩ |
| Max. Charge Current | 100 A |
| Max. Discharge Current | 150 A |
| Charge Cut-off Voltage | 58.4V |
| Discharge Cut-Off Voltage | 40V |
| Dimensions | L483mm×W650mm×H220mm |
| Series & Parallel Connection | Maxium 16 units in parallel Connection |
| Operating Temperature Range Charge Discharge Storage | 32°F (0°C) to 113°F (45°C) -4°F (-20°C) to 140°F (60°C) -4°F (-20°C) to 140°F (60°C) |
| Self-Discharge Rate | ≤3%/month @ 25°C |
| Life Expectancy (years) | 10 years @ 25°C |
| Communication Port | RS485, RS2332, CAN |

KH-ES48200

www.keheng-battery.com

DATA SHEET

Crafted out of Lithium Iron Phosphate (LiFePO4) technology, this is a battery built to last. With 3,000+ recharge cycles (and up to 7,000 @50%DOD) the 200 Ah provides 5 X the lifespan than your typical SLA battery. Built in smart BMS which can realize real-time monitoring the battery, Compatible with inverters of multiple brands, which can support 16 units parallel connection.

KEHENG 48V 200Ah LiFePO4 LITHIUM BATERY

BATTERY

SPECIFICATIONS

COMPLIED STANDARD

- IEC 62133

- UL 1642

- ISO9001

- ISO14001

- UN38.3

- RoHS

- Over charge protection function

- Over discharge protection function

- Over current protection function

- Short-circuit protection function

- Temperature protection function

- Cell Balance function

- Communicate function

- Alarm function

INTELLIGENT BMS FUNCTION

APPLICATIONS

RESS     BESS     Back-up system     Data center     Telecon     Energy storage system

Copyright © 2020 KH. All rights reserved.KH Battery Company is not liable for damages that may result from any information provided in or omitted from this publication, under any circumstances. KH Battery Company reserves the right to make adjustments to this publication at any time, without notice or obligation.

1/3

100% DOD Cycle Curves at Different Temperatures                        1C Cycle Curves at Different DOD&Temperatures

DOD in Relation to Cycle Life (25°C)                                 Discharge Time in Relation to Discharge Rate (25°C)

Temperature Effect in Relation to Battery Capacity                       Discharge Capacity in Relation to Discharge Rate

CHARACTERISTIC CURVE

2/3

Copyright © 2020 KH. All rights reserved.KH Battery Company is not liable for damages that may result from any information provided in or omitted from this publication, under any circumstances. KH Battery Company reserves the right to make adjustments to this publication at any time, without notice or obligation.

①        ② ③                ④

⑤            ⑥⑦ ⑧ ⑨ ⑩ ⑪⑫⑬⑭

①Positive Terminal             ②ADD Dialer            ③DRY CONTACT           ④Negative Terminal

⑤SS Handle                  ⑥ON/OFF Button          ⑦Reset Key               ⑧Run Indicator Light

⑨Electricity Volume Indicator    ⑩RS485 Port              ⑪CAN Port               ⑫RS232 Port

⑬RS485 Port                 ⑭RS485 Port

Copyright © 2020 KH. All rights reserved.KH Battery Company is not liable for damages that may result from any information provided in or omitted from this publication, under any circumstances. KH Battery Company reserves the right to make adjustments to this publication at any time, without notice or obligation.

BATTERY DIMENSIONS

Keheng New Energy Technology (shenzhen) Co.,Ltd

Floor 8, D8 Bldg,Lang kou Industrial Park, Long hua District, Shenzhen,china

Tel:+86 0755-21044322      	Fax: +86 0755-21044322

Website:www.keheng-battery.com

Email:info@keheng-battery.com

483mm

650mm

220mm

3/3